SECTION 05 58 13

STAINLESS STEEL COLUMN COVERS

Stainless Architectural Supply

8661 North 79th Avenue

Peoria, Arizona 85345

(623) 398-7427

www.sasmfg.com

email: sales@sasmfg.com (sales department)

This section is not necessarily all-inclusive as-is or complete for a specific project without careful editing by a technically knowledgeable person familiar with project needs and products and procedures required to properly produce intended results.

It is intended that Specifications be edited to exactly express project work, which requires inapplicable portions of the Section be deleted, and other pertinent project-specific portions added.

Red colored text shown within boxes is included for instructional use for the specifications writer and should be deleted in the final version of the Specification.

Blue text within [brackets] identifies a selection to be made. Select or revise list as required and delete the brackets in the final version of the Specification.

LEED Requirements:  Sample specification language specifying LEED requirements is shown in Green colored text within [brackets]. Delete all LEED requirements if Project is not LEED Certified.

Refer to manufacturer’s literature for technical Information for LEED MR Credits 4.1 and 4.2 for recycled content and IEQ Credit 4.1 for low-emitting materials (installation adhesive).

PART 1		GENERAL

1.01	RELATED DOCUMENTS

A.	Drawings and general provisions of the Contract, including General Conditions and Division 1 Specification Sections apply to this Section.

1.02	SUMMARY

A.	This Section includes stainless steel column covers and installation accessories.

1.03	SUBMITTALS

Delete submittals not required for Project.

A.	Product Data:  Submit manufacturer's product data for each type of column cover specified, including the following:

1.	Installation accessories.

2.	Installation instructions.

Delete the following if not necessary.

B.	Shop Drawings:  Submit shop drawings showing mounting details. Indicate field verified dimensions of each column where stainless steel column covers will be installed.

Delete reference to mock-ups if not required. Mock-ups are not typically required for column covers.

C.	Samples:  Submit samples to the Architect for review prior to delivering materials to the site or commencing the work in this Section.

Edit number of samples required to suit Project Requirements.

1.	Stainless Steel Column Covers:  Submit [3]  [ __ ] full size samples, minimum 8 inches in length, of each type of stainless steel column cover required with specified finish.

Delete the following if Project is NOT to be LEED certified. Coordinate requirements with Projects LEED requirements and Scorecard.

[D.	LEED Submittals:

1.	Credit MR 4.1 and Credit MR 4.2 – Recycled Content: Product data and certification letter indicating percentages by weight of post-consumer and pre-consumer recycled content for products having recycled content. Include statement indicating costs for each product having recycled content.

a.	Submit certification of steel process used in the manufacturing of steel items as well as the industry average recycled content per the steel recycling institute.

2.	Credit EQ 4.1 – Adhesives and Sealants:  Submit documentation including statement of VOC content.

1.04	QUALITY ASSURANCE

A.	Installer Qualifications: Installation of work of this Section shall be by a subcontractor approved by the manufacturer with a minimum of 5 years experience in similar work or other decorative metal installations having successful in-service performance.

1.05	DELIVERY, STORAGE AND HANDLING

A.	Packing and Shipping:  Deliver materials to site in manufacturer's original unopened packaging with labels intact.  Protect finished surfaces with removable wrapping or protective plastic film which will not bond when exposed to sunlight.

B.	Storage:  Store column covers in manufacturer’s original packaging, off ground and under cover away from direct sunlight, protected from weather, extreme temperature, moisture or other damage.

C.	Handling:  Handle materials so that surfaces are protected.  Prevent distortion or damage to fabricated pieces.

1.06	PROJECT/SITE CONDITIONS

Edit the following as necessary if base is installed in exterior application.

A.	Environmental Requirements:  Areas to receive column covers shall be enclosed with HVAC in operation and maintaining temperatures within range expected during building occupancy to avoid thermal dimensional change in lengths of material after installation.

1.07	SCHEDULING AND SEQUENCING

Edit or delete the following as appropriate for Project Requirements.

A.	Coordination:  Coordinate installation of stainless steel column covers with associated [stainless steel wall base] [scheduled wall base].

1.08	MAINTENANCE

Delete the following if “attic stock” maintenance materials are not required for the Project. Edit amounts / quantities as required.

A.	Extra Materials:  Furnish [2] additional column covers of each type specified in length to match units installed for Owners maintenance use.

PART 2		PRODUCTS

2.01	MANUFACTURERS

A.	Approved Manufacturer: Stainless Architectural Supply www.sasmfg.com

8661 North 79th Avenue

Peoria, Arizona 85345

(623) 398-7427.

email: sales@sasmfg.com (sales department)

2.02	MATERIALS

Type 430 or 304 stainless steel is manufacturer’s standard material. Type 316 may be available. Consult manufacturer for availability.

A.	Stainless Steel Sheet: ASTM A240, Type [430] or [304].

Column covers are available in 20 gauge thickness.

Consult manufacturer for availability of other gauges and edit if other gauge is selected.

1.	Gauge:  20 gauge.

No. 4 Brushed finish is standard. Consult manufacturer for availability of other finishes and edit if another finish is selected.

2.	Finish:  No. 4 Brushed in accordance with ASTM A480.

Delete the following if Project is NOT to be LEED certified. Coordinate percentage with Projects LEED requirements and Scorecard.

[B.	Recycled Content:  Stainless steel shall have a postconsumer recycled content plus one-half of preconsumer recycled content of not less than 25 percent.]

2.03	MANAUFACTURED CORNER GUARDS

Select model number(s) of stainless steel column cover desired for Project in either Type 430 or Type 304 stainless steel. Consult manufacturer’s current product catalog and edit as necessary.

Select the following for Type 403 stainless steel column covers.

[A.	Type 430 Stainless Steel Column Covers:]

Select desired height from standard lengths below. Custom heights are available.

[1.	Height:  48 [60] [72] [96] inches.

Edit the following to include column size(s) required (square or rectangular).

Column covers with side dimension over 12 inches are available. Column covers over 12 inches in any one side dimension require two pieces with two joints.

[2.	Column Size: [ Insert dimension ] x [ Insert dimension ].

Consult manufacturer’s catalog for column cover model number and insert below. Model numbers are based on gauge, height, and column size. Numerous models are available.

[3.	Model: [  Insert Model Number  ]

Select the following for Type 304 stainless steel column covers.

[A.	Type 304 Stainless Steel Column Covers:]

Select desired height from standard lengths below. Custom heights are available.

[1.	Height:  48 [60] [72] [96] inches.

Edit the following to include column size(s) required (square or rectangular).

Column covers with side dimension over 12 inches are available. Column covers over 12 inches in any one side dimension require two pieces with two joints.

[2.	Column Size: [ Insert dimension ] x [ Insert dimension ].

Consult manufacturer’s catalog for column cover model number and insert below. Model numbers are based on gauge, height, and column size. Numerous models are available.

[3.	Model: [  Insert Model Number  ]

2.04	INSTALLATION ACCESSORIES

Delete the following if Project is NOT to be LEED certified. Coordinate requirements with Projects LEED requirements and Scorecard. Adhesive products also comply with Greenseal GS-26, SCAQMD, CARB and NAHB Green Building Specifications.

[A.	LEED Requirements:  Adhesive shall have a VOC content of 50 g/L or less when calculated according to 40CFR 59, Subpart D (EPA Method 24).]

Select type of adhesive desired for Project.

Polymer Adhesive Coverage Rate: 160 sq. ft. using V-notch 3/16 inch x 1/4 inch x 5/16 inch trowel.

[B.	Adhesive: Manufacturer’s standard VOC compliant polymer adhesive supplied in 3.5 gallon bucket.]

[B.	Adhesive: Manufacturer’s standard polyurethane adhesive supplied in 10.5 oz or 29.0 oz tubes.]

[B.	Adhesive: Pecora 895 NST Adhesive.]

2.05	FABRICATION

A.	Field verify all dimensions of all columns to receive stainless steel column covers prior to fabrication.

B.	Stainless steel column cover shall be factory formed from stainless steel sheet with directional polish grain oriented parallel with length.

1.	Edges shall be free of burrs.

PART 3		EXECUTION

3.01	EXAMINATION

A.	Verification of Conditions:  Examine subsurfaces to receive Work and report detrimental conditions in writing to Architect.  Commencement of Work will be construed as acceptance of subsurfaces.

Revise the following two paragraphs as necessary for type of walls where stainless steel column covers will be installed.

1.	Painted walls shall be dry and thoroughly cured prior to application of stainless steel column covers.

2.	Perform adhesion test on painted drywall or plaster walls to receive stainless steel column covers in accordance with manufacturer’s installation instructions to verify proper adhesion.

B.	Coordination: Coordinate with other work which affects, connects with, or will be concealed by this Work.

1.	Coordinate installation of column covers with application of wall finishes.

2.	Verify application of wall finish is completed and ready for installation of column covers.

3.02	PREPARATION

A.	Surface Preparation: Surfaces to receive stainless steel column covers shall be cleaned to remove all dust, debris, and other contaminates that would interfere with adhesive bond.

The following is necessary to avoid dimensional change in stainless steel after or during installation process which could affect adhesive bond and proper fit of cover on column.

B.	Place stainless steel column covers in locations/rooms where units will be installed and acclimate to building conditions as specified herein in Project/Site Conditions article for a minimum period of 24 hours prior to installation.

3.03	INSTALLATION

A.	Adhesive Installation: Install stainless steel column covers with adhesive and interlocking joint in strict accordance with manufacturer’s printed installation instructions.

B.	Use tape to secure joints from movement for 24 hours after installation.

3.04	Damage and Repair

A.	Upon completion of the installation, visually check exposed surfaces of the work of this Section, and touch up scratches and damaged surfaces by field polishing so they are completely invisible to the unaided eye from a distance of five feet.

3.05	Cleaning

A.	Upon completion of installation of stainless steel corner guards, promptly clean exposed surfaces in accordance with recommendations of the stainless steel column cover manufacturer to remove all traces of dust, dirt, adhesive, and other foreign materials.

B.	During the course of the Work and on completion, remove and dispose of excess materials, equipment and debris away from premises.  Leave Work in clean condition.

END OF SECTION