SECTION 09 62 63

STAINLESS STEEL WALL BASE

Stainless Architectural Supply

8661 North 79th Avenue

Peoria, Arizona 85345

(623) 398-7427

www.sasmfg.com

email: sales@sasmfg.com (sales department)

This section is not necessarily all-inclusive as-is or complete for a specific project without careful editing by a technically knowledgeable person familiar with project needs and products and procedures required to properly produce intended results.

It is intended that Specifications be edited to exactly express project work, which requires inapplicable portions of the Section be deleted, and other pertinent project-specific portions added.

Red colored text shown within boxes is included for instructional use for the specifications writer and should be deleted in the final version of the Specification.

Blue text within [brackets] identifies a selection to be made. Select or revise list as required and delete the brackets in the final version of the Specification.

LEED Requirements:  Sample specification language specifying LEED requirements is shown in Green colored text within [brackets]. Delete all LEED requirements if Project is not LEED Certified.

Refer to manufacturer’s literature for technical Information for LEED MR Credits 4.1 and 4.2 for recycled content and IEQ Credit 4.1 for low-emitting materials (installation adhesive).

PART 1		GENERAL

1.01	RELATED DOCUMENTS

A.	Drawings and general provisions of the Contract, including General Conditions and Division 1 Specification Sections apply to this Section.

1.02	SUMMARY

Edit the following list as required.

A.	This Section includes stainless steel wall base and installation accessories of the following type:

[1.	Stainless steel cove base.]

[2.	Stainless steel straight base.]

[3.	Stainless steel sani-base.]

[4.	Stainless steel sani-base tile.]

1.03	SUBMITTALS

Delete submittals not required for Project.

A.	Product Data:  Submit manufacturer's product data for each type of wall base specified, including the following:

1.	Installation accessories.

2.	Installation instructions.

Edit reference to corners as required for project.

B.	Shop Drawings:  Submit shop drawings showing mounting details. [Include corner details [showing pre-formed corners], and end conditions.

Delete reference to mock-ups if not required. Mock-ups are not typically required for wall base but may be included as part of larger mock-up assembly showing full wall construction. Edit as required.

C.	Samples:  Submit samples to the Architect for review prior to [constructing job-site mock-ups,] delivering materials to the site or commencing the work in this Section.

Edit number of samples required to suit Project Requirements. Edit to include corners and end caps if necessary.

1.	Stainless Steel Base:  Submit [3] [ __ ] full size samples, minimum 4 inches in length, of each type of stainless steel base required with specified finish.

Delete the following if Project is NOT to be LEED certified. Coordinate requirements with Projects LEED requirements and Scorecard.

[D.	LEED Submittals:

1.	Credit MR 4.1 and Credit MR 4.2 – Recycled Content: Product data and certification letter indicating percentages by weight of post-consumer and pre-consumer recycled content for products having recycled content. Include statement indicating costs for each product having recycled content.

a.	Submit certification of steel process used in the manufacturing of steel items as well as the industry average recycled content per the steel recycling institute.

2.	Credit EQ 4.1 – Adhesives and Sealants:  Submit documentation including statement of VOC content.

1.04	QUALITY ASSURANCE

A.	Installer Qualifications: Installation of work of this Section shall be by a subcontractor approved by the manufacturer with a minimum of 5 years experience in similar work or other decorative metal installations having successful in-service performance.

1.05	DELIVERY, STORAGE AND HANDLING

A.	Packing and Shipping:  Deliver materials to site in manufacturer's original unopened packaging with labels intact.  Protect finished surfaces with removable wrapping or protective plastic film which will not bond when exposed to sunlight.

B.	Storage:  Store wall base in manufacturer’s original packaging, off ground and under cover away from direct sunlight, protected from weather, extreme temperature, moisture or other damage.

C.	Handling:  Handle materials so that surfaces are protected.  Prevent distortion or damage to fabricated pieces.

1.06	PROJECT/SITE CONDITIONS

Edit the following as necessary if base is installed in exterior application.

A.	Environmental Requirements:  Areas to receive wall base shall be enclosed with HVAC in operation and maintaining temperatures within range expected during building occupancy to avoid thermal dimensional change in lengths of material after installation.

1.07	SCHEDULING AND SEQUENCING

Edit or delete the following as appropriate for Project Requirements.

A.	Coordination:  Coordinate installation of stainless steel base with associated [stainless steel wall coverings] [stainless steel corner guards] [ceramic tile floor and wall finishes] [ ________ ] and other adjacent floor and wall finishes.

1.08	MAINTENANCE

Delete the following if “attic stock” maintenance materials are not required for the Project. Edit amounts / quantities as required.

A.	Extra Materials:  Furnish [8] lineal feet of each type of wall base specified and [2] preformed corner units of each type specified for Owners maintenance use.

PART 2		PRODUCTS

2.01	MANUFACTURERS

A.	Approved Manufacturer: Stainless Architectural Supply www.sasmfg.com

8661 North 79th Avenue

Peoria, Arizona 85345

(623) 398-7427.

email: sales@sasmfg.com (sales department)

2.02	MATERIALS

Type 430 or 304 stainless steel is manufacturer’s standard material. Type 316 may be available. Consult manufacturer for availability.

A.	Stainless Steel Sheet: ASTM A240, Type [430] or [304].

20 gauge is manufacturer’s standard for Sani-Base products. 18 gauge is standard for straight and cove base products. 16 and 18 gauge is optional for all products. Consult manufacturer for availability of other gauges and edit if other gauge is selected.

If multiple gauges are required, remove gauge from below and add the gauge to each product in each of the following product descriptions.

1.	Gauge:  [16] [18] 20 gauge.

No. 4 Brushed finish is standard. Consult manufacturer for availability of other finishes and edit if other finish is selected.

2.	Finish:  No. 4 Brushed in accordance with ASTM A480.

Delete the following if Project is NOT to be LEED certified. Coordinate percentage with Projects LEED requirements and Scorecard.

[B.	Recycled Content:  Stainless steel shall have a postconsumer recycled content plus one-half of preconsumer recycled content of not less than 25 percent.]

2.03	MANAUFACTURED WALL BASE

Select type of stainless steel base desired for Project in either Type 430 or Type 304 stainless steel. Consult manufacturer’s current product catalog and edit as necessary.

[A.	Type 430 Stainless Steel Wall Base:]

[1.	4 inch Cove Base: Model CB-4084]

[2.	4 inch Straight Base: Model SB4084]

[3.	6 inch Cove Base: Model CB-4086]

[4.	6 inch Straight Base: Model SB4086]

[A.	Type 304 Stainless Steel Wall Base:]

[1.	4 inch Cove Base: Model CB-4484]

[2.	4 inch Straight Base: Model SB4484]

[3.	6 inch Cove Base: Model CB-4486]

[4.	6 inch Straight Base: Model SB4486]

[A.	Type 430 Stainless Steel Sani-Base:]

[1.	4 inch Sani-Base Tile: Model ST3084]

[2.	6 inch Sani-Base Cove: Model SC3086]

[A.	Type 304 Stainless Steel Sani-Base:]

[1.	4 inch Sani-Base Tile: Model ST3385]

[2.	6 inch Sani-Base Cove: Model SC3386]

B.	Preformed Corner Units:  Provide preformed inside and outside corner units.

2.04	INSTALLATION ACCESSORIES

Delete the following if Project is NOT to be LEED certified. Coordinate requirements with Projects LEED requirements and Scorecard. Adhesive products also comply with Greenseal GS-26, SCAQMD, CARB and NAHB Green Building Specifications.

[A.	LEED Requirements:  Adhesive shall have a VOC content of 50 g/L or less when calculated according to 40CFR 59, Subpart D (EPA Method 24).]

Select type of adhesive desired for Project.

Polymer Adhesive Coverage Rate: 160 sq. ft. using V-notch 3/16 inch x 1/4 inch x 5/16 inch trowel.

Pecora adhesive is used for installation of Sani-Base products.

[B.	Adhesive: Manufacturer’s standard VOC compliant polymer adhesive supplied in 3.5 gallon bucket.]

[B.	Adhesive: Manufacturer’s standard polyurethane adhesive supplied in 10.5 oz or 29.0 oz tubes.]

[B.	Adhesive: Pecora 895 NST Adhesive.]

Delete below if fasteners are not used for installation. Adhesive installation is standard method of installation in most cases; however, limited use of fasteners may be helpful for secure attachment at ends and corners.

[C.	Fasteners: #10 stainless steel screws with flat pan head to be set flush.]

Specifying installation tools is not necessary; however, V-notch trowel specified below is available from manufacturer. Delete if not required.

[D.	V-Notch Trowel:  3/16 inch x 1/4 inch x 5/16 inch v-notch trowel.]

2.05	FABRICATION

A.	Stainless steel wall base and corners shall be factory formed from stainless steel sheet with directional polish grain oriented parallel with length.

1.	Edges shall be free of burrs.

2.	Standard Length: 8’-0”

3.	Fabricate exposed ends with filler to match profile of cover base.

4.	Preformed corners shall be 90 degree, or as otherwise required to suite corner degree indicated on Drawings. Fabricate preformed corners with minimum 4 inch legs.

Delete the following if base is installed with adhesive only.  Fasteners used in limited locations to reinforce attachment at corners and ends may be field drilled.

5.	Pre-drill and countersink base for seating of flush fasteners where base is indicated to be installed with fasteners.

PART 3		EXECUTION

3.01	EXAMINATION

A.	Verification of Conditions:  Examine subsurfaces to receive Work and report detrimental conditions in writing to Architect.  Commencement of Work will be construed as acceptance of subsurfaces.

Revise the following two paragraphs as necessary for type of walls where stainless steel wall base will be installed.

1.	Painted walls shall be dry and thoroughly cured prior to application of stainless steel wall base.

2.	Perform adhesion test on painted drywall or plaster walls to receive stainless steel wall base in accordance with manufacturer’s installation instructions to verify proper adhesion.

Coordinate and edit Division 03 Concrete Sections to require floor flatness tolerances within areas where stainless steel wall base is to be installed to within 1/8 inch in 10’-0”.

3.	Verify floors within areas where stainless steel wall base is to be installed are level to within 1/8 inch in 10’-0”.

B.	Coordination:  Coordinate with other work which affects, connects with, or will be concealed by this Work.

1.	Coordinate installation of base with application of wall finishes.

Revise the following two paragraphs as necessary for type of wall finishes scheduled for areas to receive stainless steel wall base.

2.	Where stainless steel base is installed after application of wall finish, verify application is completed and ready for installation of base.

3.	Install wall base prior to application of finishes installed after installation of stainless steel wall base, such as wall tile, stainless steel wall sheeting, FRP, etc.

3.02	PREPARATION

A.	Surface Preparation:  Surfaces to receive stainless steel wall base shall be cleaned to remove all dust, debris, and other contaminates that would interfere with adhesive bond.

B.	Place stainless steel base and accessory units in locations/rooms where material is to be installed and acclimate to building conditions as specified herein in Project/Site Conditions article for a minimum period of 24 hours prior to installation.

3.03	INSTALLATION

Adhesive installation is preferred method of installation for most cases.

A.	Adhesive Installation: Install stainless steel wall base with adhesive in strict accordance with manufacturer’s printed installation instructions.

B.	Install base accurately placed in proper location, level, and in proper alignment with adjoining work and applied finishes.

1.	Fit field connections accurately together to form hairline joints.

Delete the following if not required.

C.	Where necessary to provide secure attachment at corners, end terminations, or where higher abuse is anticipated, supplement adhesive installation with two (2) countersunk #10 stainless steel screws placed in pre-drilled and countersunk holes located within 1 inch of each exposed end or corner. Set screw heads flush in countersunk hole.

Delete the following if preformed corners are used. Retain if corners are field fabricated.

D.	Field Fabricated Corners: Where preformed corners are not utilized, field fabricate corners by V-notching back of base to 1/2 depth maximum of thickness of material and bend to form tightly to corner substrate.

3.04	Damage and Repair

A.	Upon completion of the installation, visually check exposed surfaces of the work of this Section, and touch up scratches and damaged surfaces by field polishing so they are completely invisible to the unaided eye from a distance of five feet.

3.05	Cleaning

A.	Upon completion of installation of stainless steel walls base, promptly clean exposed surfaces in accordance with recommendations of the stainless steel base manufacturer to remove all traces of dust, dirt, adhesive, and other foreign materials.

B.	During the course of the Work and on completion, remove and dispose of excess materials, equipment and debris away from premises.  Leave Work in clean condition.

END OF SECTION